JOBLIST DATA ENGINEERS

Pre event:

- Correlation previous track between real data and simulation (MP adapt simulation??? NOT TODAY)

- Simulation next track

- Have a clear and shared sector segmentation of the track, not to be changed during event, decide and keep it!

During event:

- Telemetry check and windarab/wintax layouts updated

  - laptime predicted need to be added (ask Max if laptime increasing channel can be configured?)

  - overlap ready to follow driving in telemetry

  - check if fuel cons calibration is the good one (have a channel with difference btw FFM and ECU calculated??? CHECK during race that the guy in charge of it have done it)

- Dash check before each session: need to check if we have good engine map, steer map, throttle map, TC maps (gain and slip)   CHECK on the car before each session (15 minutes before EVERY session)?

  - CHECK after each session and print in an outing report (not understood what we have to do in the end…)

- Telemetry while running:

  - follow reliability of the car in general

  - report performance parameters to RE:

    - Tyre pressure

    - ride height end of straight

    - caliper temp

    - brake discs temp

    - aero balance  Check the channel

    - bump rubbers squeezes  Clear shared realtime setupsheet necessary… work by changes and ask, not really functional but it should work…

    - roll distribution  Keep in mind we don’t have accelerometers on hubs, so Fz calculation won’t be really precise (Channel to be done on WinTax, update Windarab with Stefano’s channel)

    - brake bias

    - parameters depending of the driver feedback

- Post session:

  - check and report performance parameters (same at least as “telemetry while running” + potential parameters following driver feedbacks)

    - Have a clear view of parameters evolution during race (loss of downforce for pickup, suspension devices failure, EoS ride heights, damper strokes and so bump squeezes, brake bias, roll distribution, Aerobalance… ASK and DISCUSS with Cristophe what to look at)

  - driving analysis with the drivers (at least one print page per driver versus fastest driver in the team)

  - Operating limits check

Post event:

- Complete review reliability of cars

- Check and report performance parameters (potential evolution through the race)